Requirements on glacier inventory data

The inventory does not need to be strictly the same format as RGI, but it needs to be “RGI-like”. Concretely:

- We require one single file containing all glaciers in Iceland (or all glaciers that need replacing RGI7)

- The file should be shapefile or equivalent (anything that Geopandas can read, which is most of the format, including geojson or geoparquet).

- The vector geometries should all be of the type Polygon (not Multipolygon or Line). The polygons may have any number of interiors (nunataks).

- The vector geometries should be clean: no self intersection, or overlap between each other. Concretely, they should pass the test “geometry.is_valid” from Geopandas, and have no self-overlap as per the RGI test.

- The outlines should be valid at a chosen date. An obvious candidate is to match available DEM, e.g. 2010.

If we have to match observed area changes over the reanalysis period the other inventorie(s) have have the same requirements as above, with new ones:

- To match area changes, we need to know: “glacier ID 1 was x km2 in 1980, and is now y km2 in 2010”.

- For an ice cap, this implies that the same ice divides are used for both inventories and only the retreat of the outer outlines and the appearance of nunataks leads to area changes, not the changes in ice divide location (which should be a reasonable assumption).

- It implies that some form of ID system is in place (we don’t care which).

In our experience (Hartl, Schmidt et al 2025), matching area changes helps to better trust the results but have little influence on volume projections. However, having area information in 1980 for example helps to build trust in the reanalysis reconstructions as well. If we can have 2 to 3 snapshots of glacier area in Iceland, it certainly helps.